# Technical Documentation: Omnidirectional Vehicle Subsystem

Author: Dawid Tomala (Student ID: 425590)

This document outlines the theoretical side of the hardware architecture, power distribution logic, and component specifications specifically for the omnidirectional vehicle subsystem of the project. It details the design of the Mecanum-drive rover.

## 1. System Overview

The vehicle subsystem is an omnidirectional rover utilizing a Mecanum wheel drive system. The core computational unit is an ESP32-WROOM-32 (38-pin) microcontroller, which handles wireless communication and motor control logic. The propulsion system consists of four DC TT motors driven independently by two TB6612 H-bridge motor drivers. The system is powered by a 2S Li-ion battery pack (7.4V), managed by a dedicated Battery Management System (BMS) and step-down voltage regulation to safely separate logic and drive power.

## 2. Propulsion & Omnidirectional Drive System

The rover's ability to move in any direction without altering its heading relies on a carefully synchronized combination of specialized wheels, geared motors, and motor drivers.

- Mecanum Wheels: The rover utilizes four Mecanum wheels to achieve omnidirectional mobility. Unlike standard wheels, Mecanum wheels feature passive rollers angled at 45 degrees around their circumference. By independently controlling the rotational speed and direction of each of the four wheels, the resulting force vectors can be combined to move the rover forward, backward, strafe directly sideways, move diagonally, or rotate in place.

- DC Gear Motors (TT Motors): Each Mecanum wheel is directly driven by an independent DC "TT" gear motor. These motors feature an internal reduction gearbox, providing the necessary torque required to overcome the complex lateral friction forces generated by the angled rollers during omnidirectional maneuvers.

- TB6612 Motor Drivers: To independently control the four motors, the system uses two TB6612 dual H-bridge motor driver modules (each module handling two motors). The TB6612 utilizes MOSFET-based H-bridges, which makes it significantly more energy-efficient and prevents the severe voltage drops commonly seen in older, BJT-based drivers like the L298N. The drivers interpret directional logic signals (IN1, IN2) from the ESP32 to set rotation direction and accept PWM signals to precisely modulate the speed of each wheel, which is a critical requirement for calculating accurate Mecanum kinematics.

## 3. Power Management & Distribution Architecture

Proper power distribution is critical to isolate the sensitive logic circuitry from the high-current demands and electrical noise of the drive motors.

- Main Power Source: A 2S Li-ion battery pack provides a nominal 7.4V (up to 8.4V fully charged). It is protected by a 2S 8A BMS and can be charged directly inside the rover via an IP2326 Charging Module (USB-C) wired in parallel to the battery cells.

- Power Switch: An E-TEN1021 main toggle switch is placed in series on the positive line (P+) immediately after the BMS. Throwing this switch simultaneously cuts off power to both the logic step-down converter and the motor drivers, ensuring zero power draw when the robot is inactive.

- Motor Power (VM): The raw 7.4V from the output of the BMS is routed directly to the high-current VM pins on both TB6612 drivers. Since the TT motors are nominally rated for 3-6V, supplying a continuous 7.4V would risk thermal damage. To resolve this without adding an additional step-down converter, voltage regulation will be managed entirely in software via Pulse Width Modulation (PWM). By capping the maximum PWM duty cycle in the ESP32 firmware to approximately 75-80%, the average voltage delivered to the motor coils will safely remain below 6.0V, ensuring optimal torque while protecting the hardware..

- Logic Power (5V & 3.3V): The 7.4V line is regulated down to a stable 5V by an MP1584EN Step-Down Converter. This 5V output feeds the VIN pin of the ESP32. The ESP32's internal low-dropout regulator (LDO) then provides a stable 3.3V reference (via the 3V3 pin), which is used to power the logic circuitry (VCC) and wake up the motor drivers (STBY).

- Common Ground Topology: A unified ground net connects the BMS P-, the MP1584EN OUT-, the ESP32 GND, and all TB6612 GND pins. This ensures stable reference voltages for PWM/logic signals and provides a safe return path for high motor currents.

## 4. Component Documentation & Datasheets

Below is the list of all major hardware components used in the vehicle subsystem. Links to the official datasheets, technical specifications, and vendor documentation for each component will be maintained here for reference during assembly and troubleshooting.

### Microcontroller & Logic

- ESP32-WROOM-32 (38-pin Development Board)

  - Function: Main processor, wireless communication, and motor PWM generation.

  - Documentation:

- https://documentation.espressif.com/esp32-wroom-32_datasheet_en.pdf

- https://www.xecor.com/blog/esp32-pinout-diagram

### Drive System

- TB6612FNG Dual Motor Driver (x2)

  - Function: H-bridge drivers controlling the speed and direction of the four DC motors.

  - Documentation:

- https://cdn.sparkfun.com/datasheets/Robotics/TB6612FNG.pdf

- https://lastminuteengineers.com/tb6612fng-dc-motor-control-arduino-tutorial/

- TT Gear Motor 3-6V DC 125RPM (x4)

  - Function: Actuators driving the Mecanum wheels.

  - Documentation:

- https://cdn.shopify.com/s/files/1/0837/4340/8415/files/shillehtek-ad092-datasheet.pdf?v=1790027048

- https://www.avmarket.eu/img/cms/69-572-4_1.jpg

  Mecanum Wheels (x4)

- Function: Specialized wheels with 45-degree passive rollers that allow for omnidirectional maneuverability.

- Documentation:

- https://handsontec.com/dataspecs/robotics/Mecanum%20Wheel.pdf

- https://research.ijcaonline.org/volume113/number3/pxc3901586.pdf

### Power & Battery Management

- 2S Li-ion Battery Pack (Samsung 30Q cells)

  - Function: Main power storage for the rover.

  - Documentation:

- https://e2e.ti.com/cfs-file/__key/communityserver-discussions-components-files/196/INR18650_2D00_30Q_5F00_datasheet.PDF

- 2S 8A Battery Management System (BMS)

  - Function: Overcharge, over-discharge, and short-circuit protection for the Li-ion cells.

  - Documentation:

- https://www.tztstore.com/goods/show-4669.html

- IP2326 Charging Module (USB-C)

  - Function: Step-up charging and cell balancing directly from a 5V USB-C source.

  - Documentation:

- https://done.land/assets/files/ip2326_datasheet.pdf

- https://done.land/components/power/powersupplies/battery/chargers/charge/powermanagementics/ip2326/

- MP1584EN Step-Down Converter

  - Function: Buck converter dropping the 7.4V battery voltage to 5V for the ESP32 logic.

  - Documentation:

- https://www.monolithicpower.com/en/documentview/productdocument/index/version/2/document_type/Datasheet/lang/en/sku/MP1584EN-LF-Z/

- https://sparks.gogo.co.nz/assets/_site_/downloads/MP1584_Buck_Module.pdf

- https://www.amateurfunk-sulingen.de/forum/attachments/MP1584-schematic.pdf

- E-TEN1021 Toggle Switch

  - Function: Main system power cutoff.

  - Documentation:

- https://www.hammerheadindustrial.com/products/mini-toggle-switch-e-ten1021-2-position-2-pin-on-off-ac-250v-15a-toggle-switch

For point-to-point electrical connections, GPIO assignments, and detailed wiring, refer to the accompanying Pinout Configuration document.