# Hardware Architecture & Connection Wiring: Omnidirectional Vehicle Subsystem

Author: Dawid Tomala (Student ID: 425590)

## This document outlines the complete hardware architecture and connection wiring specifically for the omnidirectional vehicle subsystem of the project, detailing the electrical schematic, power distribution, and GPIO pinout configurations required for the Mecanum-drive rover.

## 1. ESP32-WROOM-32 Connections

| ESP32 Pin | Pin Name | Destination | Function in the System |
| --- | --- | --- | --- |
| Pin 1 | 3V3 | STBY (Pin 4) & VCC (Pin 15) on both TB6612s | Wakes up the H-bridges and supplies 3.3V to their logic circuitry. |
| Pin 11 | GPIO27 | BIN1 (Pin 5) - Bottom TB6612 (Right Side) | Direction 1 control for Rear-Right motor. |
| Pin 12 | GPIO14 | BIN2 (Pin 6) - Bottom TB6612 (Right Side) | Direction 2 control for Rear-Right motor. |
| Pin 13 | GPIO12 | PWMB (Pin 7) - Bottom TB6612 (Right Side) | PWM (speed) signal for Rear-Right motor. |
| Pin 19 | VIN | OUT+ (Pin 8) on MP1584EN | Main 5V power input for the microcontroller from the step-down converter. |
| Pin 26 | GPIO4 | AIN1 (Pin 3) - Bottom TB6612 (Right Side) | Direction 1 control for Front-Right motor. |
| Pin 27 | GPIO16 | AIN2 (Pin 2) - Bottom TB6612 (Right Side) | Direction 2 control for Front-Right motor. |
| Pin 28 | GPIO17 | PWMA (Pin 1) - Bottom TB6612 (Right Side) | PWM (speed) signal for Front-Right motor. |
| Pin 29 | GPIO5 | PWMB (Pin 7) - Top TB6612 (Left Side) | PWM (speed) signal for Rear-Left motor. |
| Pin 30 | GPIO18 | BIN2 (Pin 6) - Top TB6612 (Left Side) | Direction 2 control for Rear-Left motor. |
| Pin 31 | GPIO19 | BIN1 (Pin 5) - Top TB6612 (Left Side) | Direction 1 control for Rear-Left motor. |
| Pin 33 | GPIO21 | AIN1 (Pin 3) - Top TB6612 (Left Side) | Direction 1 control for Front-Left motor. |
| Pin 36 | GPIO22 | AIN2 (Pin 2) - Top TB6612 (Left Side) | Direction 2 control for Front-Left motor. |
| Pin 37 | GPIO23 | PWMA (Pin 1) - Top TB6612 (Left Side) | PWM (speed) signal for Front-Left motor. |
| Pin 38 | GND | Common GND net | Connects to the converter's ground, BMS ground, and all motor driver ground pins. |

## 2. Power & Drive Connections

| Component | Power Pin | Destination | Description |
| --- | --- | --- | --- |
| BMS 2S 8A | P+ (Pin 4) | Power Switch (Pin 1) | Main positive power output (7.4V) from the battery. |
| BMS 2S 8A | P- (Pin 2) | MP1584EN IN- (Pin 4) | Ground return path for the entire robot back to the battery. |
| Power Switch | Output (Pin 2) | MP1584EN IN+ (Pin 1) & VM on both TB6612s (Pin 16) | Main node distributing 7.4V to the motor coils and the logic step-down converter. |
| MP1584EN | OUT- (Pin 5) | System GND net | Point connecting the battery power circuit with the common ground of the microcontroller and drivers. |
| TB6612 (Both) | GND (Pins 8, 9, 14) | System GND net (ESP32 Pin 38 / MP1584EN OUT-) | Return path for currents to the main ground. |

## 3. Schematic diagram